FIPS 123 Function Library

Version 1.0.3 3/11/94

Current Discrepancy Reports / Statements of Need

This is a summary of all existing discrepancy reports and statements

of need (enhancement requests) for the FIPS 123 Function Library.

These will be fixed in future versions as time permits.

92DR001 11/25/91              No DDR in memory

If a DDR does not reside in memory and a field or

subfield is erased using er123ddfld() or

er123ddsfld(), both functions return failure.

Note er123ddrec() does not fail under the same

instances.

92DR004   /  /                Back up past 1st rec

If an application tries to back up beyond first

data record in a transfer file, the "BAK"

functions return a failure.  This does not allow

the calling application routine to recover.

92DR007 03/19/92              bak123sfld

Invoking bak123sfld() after reading a record or

field does not leave the application program in

the correct subfield location for subsequent

reads.

92SN003   /  /                Error handling

Design, code, and implement error handling for the

FIPS123 function library and software.

93DR020 01/28/93              Max unsigned integer

The FIPS PUB 123 Function Library defined constant

MAXSIZ determines the maximum amount of contiguous

memory space that can be allocated during

processing.  MAXSIZ is defined as 64000 for the

DOS environment and 128000 for the other

environments.

The value of MAXSIZ may cause failure in routines

that pass parameters of type size_t.

The predefined type, size_t, is defined in ANSI C

as an unsigned integer.  The compilers currently

used within the MS-DOS, DG/UX, and MVS

environments all conform to this requirement.

However, ANSI C defines the maximum size of an

unsigned integer to be two bytes (16 bits),

capable of storing a maximum value of 65,535

(64K-1).  MS-DOS conforms to this standard, but

DG/UX and MVS environments allocate 32 bits for

unsigned integers, capable of storing the maximum

value of 4,294,967,295.

It is highly likely that future and currently

untested environments will conform to the ANSI C

convention that unsigned integers can not store

values greater than 65,535.  In this case, the

value of MAXSIZ (128,000) will overflow when

passed to arguments declared in the FIPS PUB 123

Function Library as type size_t (unsigned

integer).

93DR022 01/29/93              Verify level 3 tags

Although three previous discrepancy reports

(92DR002, 92DR003, and 92DR006) describing the

verification of the order of field tags in the DDR

and DRs have been completed, there remains a more

detailed change that is specific to level 3

interchange files.

Level 3 files contain compound data fields and a

list of tag pairs describing hierarchical

structures.  These structures are absent in level

1 and 2 files.  The order of the DDR field tags

for level 3 files should be in the same order as

the pre-order traversal sequence of the data tree.

The order of the DR field tags are ordered

similarly.

Additional information is requested from the USGS

SDTS Task Force in order to implement tag order

verification for level 3 files correctly.

The discrepancies listed above have been

implemented to check tag order for level 1 and 2

files.  No verification occurs for level 3 files

at this time.

93DR024 03/05/93              Use of ch123size

Function end123file calls ch123size which copies

the entire file to a new file, truncating the file

at its current position.  This is necessary if the

program has called wr123rec or the backing

functions but is not necessary if the program has

only called wr123fld or wr123sfld as is done by

the DLG-3 to SDTS programs.  This reading of the

original file and writing of a second file can

potentially degrade performance when writing

files.

93DR026 03/12/93              Write funct. status

Functions wr123ddrec() and wr123rec() contain the

argument status within its calling parameters.

These functions return a status of 'failure' (0)

if an error is encountered when processing a

record.  If no error occurs, the named functions

do not change the value of status; thus, the value

returned is the same as the value passed into the

functions.

To provide a consistent return status value, only

two values should be returned:  'failure' (0) upon

an error, and 'okay' (1) when the record is

written successfully.

93DR028 04/02/93              Pad with ^ character

A revision to ISO8211 has been made.  This

revision pads the end of transfer files with a '^'

character to ensure the file ends at a certain

byte count.  For additional information consult

5th year revised ISO8211.

93DR029 04/21/93              Erase DDR functions

The 'erase' functions are responsible for erasing

DDR subfields and fields and a complete DDR for a

file opened in write mode.  These functions should

only be invoked prior to processing a data record.

If an erase function is invoked after initiating

DR processing, unpredictable results may occur.

The results are dependent upon the type of

processing performed by the application program.

Currently, there is no logic that signals the

invocation of an erase function after initiating

DR processing.  Thus, an application may continue

to execute, creating an invalid data file, until

an error is encountered severe enough to halt

execution.

The erase functions should be modified to return

failure if invoked after initiating DR processing.

93DR041 10/27/93              ch_size rename

In ch_size.c, use of rename function on temporary

file fails if user is creating destination output

files in a file system different from current

directory since the rename function cannot work

accross file systems.  The result is temp files

accumulate and are not removed and data doesn't go

to where it should.  No errors are reported by the

software.

Discovered on Sun platform at USA/CERL.

93DR042 12/17/93              EBCDIC to ASCII

The FIPS PUB 123 Function Library currently

reads and writes a data file in the character

set of the machine on which the software is

executing.  Applications executing in DOS

and UNIX read, process, and write a data

files in the ASCII character set, while MVS

reads, processes, and writes data files in

the EBCDIC character set.  A utility, DDFCONV,

converts data files from one character set to

the other.  The production of DLGs in SDTS

format would benefit from the ability of the

FIPS PUB 123 Function Library to read or write

either data set.  Applications under MVS could

write a DLG in SDTS format as an ASCII file and

avoid running the DDFCONV utility.

#&a6H

#